Motor Sync Logic Recommendations.

Okay, let's dive deep into the motor synchronization logic, analyze why it might be failing, and suggest improvements.

1. Analysis of the Current Synchronization Logic

The synchronization relies on these components:

- Feedback: Motor_FB_Reads() uses polling (triggered every 500us by TIM14 setting MOTOR_FB_READS_FLAG) to read GPIOs connected to motor feedback sensors (likely Hall effect or encoders). It uses an 8-bit shift register for basic debouncing. On detecting a falling edge (state == 0x80), it increments/decrements a float counter[] based on the current direction[].

- Position Normalization: normalizedPulseCount[x] = (float)counter[x] / PULSE_TOTAL_MOTORx. This converts the raw pulse count into a 0.0 to ~1.0 range.

- Limit Stopping: Motor_FB_Reads() also sets direction[x] = 0 if the counter[x] reaches <= 0 or >= PULSE_TOTAL_MOTORx.

- State Transitions: Motor_FB_Reads() triggers the transition to LIFT_EXTENDED or LIFT_RETRACTED only when both motors have direction == 0 and their counters are at the respective limits.

- Control Algorithm (AdjustSpeed()):

  - It's a Proportional (P) controller.

  - It determines which motor is leading/lagging based on direction and normalizedPulseCount.

  - Calculates the absolute error: error = fabs(normalizedPulseCount[laggingIndex] - normalizedPulseCount[leadingIndex]).

  - Adjusts the leading motor's speed: adjustedSpeed = DESIRED_SPEED - (int)(error * K_P). The result is clamped to be >= 0.

  - Sets the lagging motor's speed to DESIRED_SPEED.

  - Uses K_P = 12000.

  - Calls driveActuator() for both motors in every main loop cycle where CurrentOpState < TEST_EXTEND.

2. Why Actuators Might Become Unsynchronized

Several factors in this implementation could lead to desynchronization:

- Feedback Errors (Missed or False Pulses): This is the most likely culprit.

  - Polling Rate vs. Pulse Width: The feedback state is checked only when the MOTOR_FB_READS_FLAG block executes in the main loop. While the flag is set every 500us, the actual time between checks depends on how long the rest of the main loop takes. If a feedback pulse is shorter than the time between reads, or if the main loop execution time varies, pulses can be missed.

  - Debouncing: The 8-sample shift register debounce (state == 0x80) requires the signal to be stable low for 8 consecutive samples (8 * 500us = 4ms sampling window for the edge). Noise during this window or pulses faster than this could cause missed falling edges or false triggers.

  - Impact: If one motor's counter becomes inaccurate due to missed/false pulses, AdjustSpeed receives incorrect position information. It will then command incorrect speed adjustments, potentially driving the motors further out of sync instead of correcting. If counts are significantly off, one motor might hit its physical end stop while the code thinks it still has distance to travel, allowing the other motor to continue running independently until its (potentially inaccurate) counter also reaches its limit.

- High Proportional Gain (K_P = 12000):

  - Calculation: Let's analyze the adjustment (int)(error * K_P). With PULSE_TOTAL = 460, a single pulse difference is an error of 1/460 ≈ 0.00217. The adjustment is (int)(0.00217 * 12000) ≈ (int)(26.04) = 26.

  - Impact: A tiny error (even just 1-2 pulses) causes a significant reduction (26-52%) in the leading motor's speed. An error of just 4 pulses (4/460 ≈ 0.0087, adjustment (int)(0.0087 * 12000) = 104) is enough to completely stop the leading motor (100 - 104 -> 0).

  - Result: This extremely high gain makes the system hyper-sensitive. Instead of smooth corrections, it likely causes the leading motor to frequently stop or slow drastically, letting the lagging motor surge ahead, potentially overshooting and then becoming the leader itself. This can lead to jerky motion, oscillations, and doesn't effectively manage synchronization if the underlying cause (like feedback errors) isn't addressed. It can look like independent operation as one motor constantly starts and stops aggressively while the other runs full speed.

- No Deadband:

  - Issue: The controller attempts to correct any non-zero error, no matter how small. Given potential minor variations in sensors or floating-point calculations, this can lead to constant, unnecessary speed adjustments when the motors are actually acceptably synchronized.

  - Impact: Contributes to jerky movement and potential instability, especially combined with the high gain.

- Limit Handling and State Transitions:

  - Issue: A motor's direction is set to 0 only after the feedback pulse indicating the limit has been reached is processed by Motor_FB_Reads. The transition to LIFT_EXTENDED/RETRACTED only happens when both directions are 0 and counters are at limits.

  - Impact: If one motor (A) hits its limit, its direction becomes 0. AdjustSpeed will still run as long as direction[B] is non-zero. It will continue trying to adjust speeds based on the now-static position of A and the moving position of B. Motor B will continue until its limit-reaching pulse is processed. While not truly independent control, one stops significantly before the other if they were out of sync approaching the limit. If feedback errors prevent one motor's counter from ever reaching the limit, the state transition might never occur, potentially leaving one motor stopped at its physical limit while the code keeps trying to run the other.

- Floating Point Counters:

  - Issue: Using float for counter and prevCounter is unnecessary and potentially problematic. Pulse counts are inherently integers. Floating-point comparisons (>=, <=) can sometimes have subtle precision issues, although less likely to be the primary cause here than feedback errors or high gain.

  - Recommendation: Use int32_t or uint32_t for pulse counts.

- Initialization (counter = 600) & Stall Detection:

  - Issue: The initial forced retraction relies on hitting the end stops hard enough to physically stall both motors near the zero point, after which the code sets counter = 0 in the LIFT_RETRACTED state. If this process isn't perfectly reliable (e.g., stall detection triggers early due to friction/intermittent pulses), the initial counter = 0 state might not match physical reality. The stall detection itself (prevCounter == counter checked every 1.5s) is also susceptible to intermittent missed pulses.

  - Impact: Starting an extension cycle with incorrect initial position counts guarantees synchronization problems.

3. Detailed Recommendations for Improvement:

- Overhaul Feedback Mechanism (Highest Priority):

  - Switch to External Interrupts (EXTI): This is the standard, most robust way to count pulses.

    - Configure MOTOR1_FEEDBACK_Pin and MOTOR2_FEEDBACK_Pin as inputs with interrupts enabled on the appropriate edge (e.g., GPIO_MODE_IT_FALLING).

    - Implement HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin).

    - Inside the callback, identify which pin triggered it. If it's a motor feedback pin, increment or decrement the corresponding counter based on the current direction. Keep this ISR extremely short.

    - Declare the counter variables (motor_pulse_count) as volatile int32_t.

    - Remove TIM14, MOTOR_FB_READS_FLAG, and the Motor_FB_Reads() function entirely.

    - Benefit: Eliminates missed pulses due to polling rate, more efficient, more accurate counting.

- Refine Synchronization Algorithm (AdjustSpeed):

  - Drastically Reduce K_P: Start with a much smaller value (e.g., K_P = 500) and tune experimentally by observing the physical behavior. Increase it gradually until you get responsive but smooth correction without oscillation or abrupt stopping. The value 12000 is almost certainly too high.

  - Introduce a Deadband: Only apply corrections if the error is significant.

C

#define SYNC_ERROR_DEADBAND_PERCENT 0.5 // Example: 0.5% tolerance

#define SYNC_ERROR_DEADBAND_NORMALIZED (SYNC_ERROR_DEADBAND_PERCENT / 100.0)

// ... calculate error ...

if (error > SYNC_ERROR_DEADBAND_NORMALIZED && leadingIndex != -1) {

// Apply P-control

int adjustment = (int)(error * K_P_SYNC); // Use NEW, TUNED K_P_SYNC

int adjustedSpeed = DESIRED_SPEED - adjustment;

// Apply clamping (maybe ensure minimum drive speed?)

speed[leadingIndex] = adjustedSpeed >= MIN_MOTOR_DRIVE_SPEED ? adjustedSpeed : MIN_MOTOR_DRIVE_SPEED;

speed[laggingIndex] = DESIRED_SPEED;

} else {

// Within deadband: Set both to desired speed (if moving)

speed[MOTOR_LEFT] = (direction[MOTOR_LEFT] != 0) ? DESIRED_SPEED : 0;

speed[MOTOR_RIGHT] = (direction[MOTOR_RIGHT] != 0) ? DESIRED_SPEED : 0;

}

  - Consider Minimum Speed: Instead of clamping adjustedSpeed to 0, maybe clamp it to MIN_MOTOR_DRIVE_SPEED (e.g., 25) unless the error is extremely large. This prevents the leading motor from stopping completely for small corrections.

  - Use Integer Counters: Change counter, prevCounter to volatile int32_t. Perform normalization to float locally within AdjustSpeed just before calculating the error.

- Improve Limit Handling:

  - Check Limits Frequently: Since feedback is now handled by EXTI, the main loop needs to check motor_pulse_count against 0 and PULSE_TOTAL_MOTORx frequently to know when to stop a motor. Create a helper function CheckMotorLimits() called in every loop iteration.

  - Immediate Stop: When a limit is reached in CheckMotorLimits(), immediately call MotorX_Stop() and set direction[x] = 0.

  - State Transitions: Base state transitions (LIFT_EXTENDED/RETRACTED) on both direction variables being 0 (as determined by CheckMotorLimits).

  - (Optional) Add Timeouts: Add failsafe timeouts to LIFT_EXTENDING and LIFT_RETRACTING states. If the motors run for too long (e.g., 1.5x expected time) without the CheckMotorLimits function detecting the end state, transition to a LIFT_FAULT state and stop everything.

- Refine Initialization:

  - Keep the initial forced retraction to physically synchronize at the bottom.

  - After this sequence seems complete (e.g., based on a timeout or monitoring the DRV8873 fault/current sense pins if implemented), explicitly reset the EXTI-updated motor_pulse_count variables to 0.

By implementing EXTI for feedback and carefully tuning a lower K_P gain with a deadband, you should see significantly smoother and more reliable motor synchronization.