DRV8873 Current Sensing

Okay, let's add code to monitor motor current using the DRV8873's capabilities, specifically targeting the SPI interface and built-in Overcurrent Protection (OCP), which is the most integrated way for the DRV8873S variant.

This involves:

- Configuring OCP via SPI: Setting the DRV8873's internal threshold near 3A during initialization.

- Monitoring Faults: Using the nFAULT pin (via GPIO EXTI) and reading the DRV8873's fault registers via SPI.

- Fault Handling: Stopping motors and entering a dedicated fault state if an OCP event occurs.

Assumptions:

- You have a working SPI interface configured (hspi1 perhaps?) capable of communicating with both DRV8873 devices (using separate Chip Select lines).

- You have helper functions in motor.c (or similar) to handle the low-level SPI read/write operations for each driver (e.g., Motor1_Write_Register, Motor1_Read_Register, Motor2_...). These need to manage Chip Select and the specific SPI protocol defined in the DRV8873 datasheet.

- The nFAULT pins from each DRV8873 are connected to separate STM32 GPIO pins (e.g., MOTOR1_nFAULT_Pin, MOTOR2_nFAULT_Pin) configured as Inputs with Pull-ups and triggering External Interrupts (EXTI) on a Falling Edge.

Code Modifications:

1. Add Defines and Flags:

C

// In app.h or near the top of app.c

// ... other flags ...

#define MOTOR_FAULT_FLAG 0x8UL          // Existing general fault flag

#define MOTOR_OVERCURRENT_FAULT_FLAG 0x20UL // New flag specific to OCP

#define MOTOR1_FAULT_CHECK_PENDING 0x40UL // Flag: Check Motor 1 fault register

#define MOTOR2_FAULT_CHECK_PENDING 0x80UL // Flag: Check Motor 2 fault register

// Add more flags if needed, ensure no overlap

// --- DRV8873 SPI Register/Bit Definitions (EXAMPLE - VERIFY WITH DATASHEET!) ---

// These need to match the actual DRV8873 datasheet

#define DRV8873_REG_FAULT 0x00 // Example Fault Status Register Address

#define DRV8873_REG_CTRL1 0x01 // Example Control Register 1 Address (might contain OCP settings)

#define DRV8873_REG_CTRL2 0x02 // Example Control Register 2 Address (might contain OCP settings)

// Example OCP Mode bits within a control register (e.g., CTRL2)

#define DRV8873_OCP_MODE_MASK 0x0C      // Mask for OCP bits (e.g., bits 3:2)

#define DRV8873_OCP_MODE_3A0  (0x02 << 2) // Hypothetical setting for 3.0A (Check Datasheet!)

#define DRV8873_OCP_DEG_MASK  0x03      // Mask for OCP Deglitch Time bits (e.g., bits 1:0)

#define DRV8873_OCP_DEG_10US  (0x01 << 0) // Hypothetical setting for 10.6us (Check Datasheet!)

// Example Fault Register Bits (e.g., FAULT Register 0x00)

#define DRV8873_FAULT_OCP_HA  (1 << 5)  // OCP on High-Side A

#define DRV8873_FAULT_OCP_LA  (1 << 4)  // OCP on Low-Side A

#define DRV8873_FAULT_OCP_HB  (1 << 3)  // OCP on High-Side B

#define DRV8873_FAULT_OCP_LB  (1 << 2)  // OCP on Low-Side B

#define DRV8873_FAULT_OTS     (1 << 1)  // Overtemperature Shutdown

#define DRV8873_FAULT_UVLO    (1 << 0)  // Undervoltage Lockout

// Mask for any OCP fault

#define DRV8873_FAULT_BITS_OCP (DRV8873_FAULT_OCP_HA | DRV8873_FAULT_OCP_LA | DRV8873_FAULT_OCP_HB | DRV8873_FAULT_OCP_LB)

// --- End DRV8873 Definitions ---

#define OVERCURRENT_THRESHOLD_AMPS 3.0f // For reference / ADC method if used

2. Update LiftOperationState Enum:

C

typedef enum

{

LIFT_INIT,

LIFT_EXTENDING,

LIFT_EXTENDED,

LIFT_EXTENDED_WAIT,

LIFT_PRE_RETRACTING,

LIFT_RETRACTING,

LIFT_RETRACTED,

LIFT_FAULT, // <-- Add Fault State

TEST_EXTEND,

TEST_RETRACT,

TEST_STOP

} LiftOperationState;

3. Modify MotorInit (Conceptual - Resides likely in motor.c):

C

// In motor.c (or wherever motor hardware init occurs)

// Assumed SPI Read/Write functions declared (e.g., in motor.h)

// HAL_StatusTypeDef Motor1_Write_Register(uint8_t reg_addr, uint8_t data);

// HAL_StatusTypeDef Motor2_Write_Register(uint8_t reg_addr, uint8_t data);

// HAL_StatusTypeDef Motor1_Read_Register(uint8_t reg_addr, uint8_t* data);

// HAL_StatusTypeDef Motor2_Read_Register(uint8_t reg_addr, uint8_t* data);

void MotorInit(void)

{

// Initialize GPIOs for IN1/IN2/PWM/nSLEEP/ChipSelects etc.

// Initialize Timers for PWM if using PWM mode controlled via SPI

// Initialize SPI peripheral used for DRV8873 communication

// MX_SPIx_Init(); // Assuming CubeMX generated function

// --- Configure DRV8873 OCP via SPI ---

// IMPORTANT: Read the existing register value first, modify only the

// necessary bits, then write it back. Avoid overwriting other settings.

uint8_t ctrl_reg_val = 0;

uint8_t new_ctrl_reg_val = 0;

HAL_StatusTypeDef status;

// Configure Motor 1 OCP (Example using CTRL2 register)

status = Motor1_Read_Register(DRV8873_REG_CTRL2, &ctrl_reg_val);

if (status == HAL_OK)

{

new_ctrl_reg_val = (ctrl_reg_val & ~DRV8873_OCP_MODE_MASK) | DRV8873_OCP_MODE_3A0; // Set OCP ~3A

new_ctrl_reg_val = (new_ctrl_reg_val & ~DRV8873_OCP_DEG_MASK) | DRV8873_OCP_DEG_10US; // Set Deglitch

status = Motor1_Write_Register(DRV8873_REG_CTRL2, new_ctrl_reg_val);

if (status != HAL_OK) { /* Handle SPI Write Error */ }

} else { /* Handle SPI Read Error */ }

// Configure Motor 2 OCP (Example using CTRL2 register)

status = Motor2_Read_Register(DRV8873_REG_CTRL2, &ctrl_reg_val);

if (status == HAL_OK)

{

new_ctrl_reg_val = (ctrl_reg_val & ~DRV8873_OCP_MODE_MASK) | DRV8873_OCP_MODE_3A0; // Set OCP ~3A

new_ctrl_reg_val = (new_ctrl_reg_val & ~DRV8873_OCP_DEG_MASK) | DRV8873_OCP_DEG_10US; // Set Deglitch

status = Motor2_Write_Register(DRV8873_REG_CTRL2, new_ctrl_reg_val);

if (status != HAL_OK) { /* Handle SPI Write Error */ }

} else { /* Handle SPI Read Error */ }

// Ensure drivers are enabled (e.g., nSLEEP pin high or relevant SPI bit set)

// ...

// Clear any initial faults (optional, check datasheet procedure)

Motor1_Write_Register(DRV8873_REG_FAULT, 0xFF); // Writing often clears latched faults

Motor2_Write_Register(DRV8873_REG_FAULT, 0xFF);

// Initialize nFAULT GPIOs and EXTI in main.c/stm32g0xx_hal_msp.c

// MX_GPIO_Init();

}

// Optional: Function to clear faults (if needed for recovery)

HAL_StatusTypeDef Motor_Clear_Faults(uint8_t motor_idx) {

// Datasheet specifies how to clear faults (e.g., read fault register,

// write back to fault register, toggle nSLEEP, or write specific clear bit)

uint8_t fault_status = 0;

HAL_StatusTypeDef status = HAL_ERROR;

if (motor_idx == MOTOR1_IDX) {

// Example: Reading FAULT register might clear it, or writing back

status = Motor1_Read_Register(DRV8873_REG_FAULT, &fault_status);

// Or status = Motor1_Write_Register(DRV8873_REG_FAULT, 0xFF);

} else if (motor_idx == MOTOR2_IDX) {

status = Motor2_Read_Register(DRV8873_REG_FAULT, &fault_status);

// Or status = Motor2_Write_Register(DRV8873_REG_FAULT, 0xFF);

}

return status;

}

4. Implement nFAULT Interrupt Handler (stm32g0xx_it.c):

C

// In stm32g0xx_it.c

#include "main.h" // Ensure flags are visible

#include "app.h"  // Include app header for flags if defined there

// Assumes MOTOR1_nFAULT_Pin and MOTOR2_nFAULT_Pin are defined in main.h

// Assumes these pins are configured for EXTI falling edge trigger

extern uint32_t app_flags; // Make app_flags accessible

void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

if (GPIO_Pin == MOTOR1_nFAULT_Pin)

{

// nFAULT asserted on Motor 1 - Could be OCP, UVLO, OTS

// Set flag for main loop to check details via SPI

ATOMIC_SET_BIT(app_flags, MOTOR_FAULT_FLAG | MOTOR1_FAULT_CHECK_PENDING);

// Optional: Disable EXTI line temporarily to prevent re-triggering until handled

// HAL_NVIC_DisableIRQ(EXTIx_IRQn); // Replace x with correct EXTI line number

}

else if (GPIO_Pin == MOTOR2_nFAULT_Pin)

{

// nFAULT asserted on Motor 2

ATOMIC_SET_BIT(app_flags, MOTOR_FAULT_FLAG | MOTOR2_FAULT_CHECK_PENDING);

// Optional: Disable EXTI line

// HAL_NVIC_DisableIRQ(EXTIy_IRQn);

}

// Handle other GPIO pin interrupts if any

// else if (GPIO_Pin == ...) { ... }

}

// Remember to implement the corresponding EXTIx_IRQHandler in stm32g0xx_it.c

// which calls HAL_GPIO_EXTI_IRQHandler(GPIO_PIN_x); which in turn calls the Callback.

// e.g.

// void EXTI4_15_IRQHandler(void) {

//    HAL_GPIO_EXTI_IRQHandler(MOTOR1_nFAULT_Pin);

//    HAL_GPIO_EXTI_IRQHandler(MOTOR2_nFAULT_Pin);

//    // Add other pins handled by this IRQ handler

// }

5. Modify Main Loop (AppStart in app.c):

C

// In app.c

// Include motor.h if SPI functions are declared there

#include "motor.h"

// --- Add this function ---

/**

* @brief Checks DRV8873 fault status via SPI and handles Overcurrent faults.

* @return bool True if an Overcurrent Fault was detected and handled, false otherwise.

*/

static bool CheckAndHandleOvercurrentFault(void)

{

bool oc_fault_detected = false;

uint8_t fault_status = 0;

HAL_StatusTypeDef spi_status;

// Check Motor 1 if pending

if (READ_BIT(app_flags, MOTOR1_FAULT_CHECK_PENDING))

{

spi_status = Motor1_Read_Register(DRV8873_REG_FAULT, &fault_status);

ATOMIC_CLEAR_BIT(app_flags, MOTOR1_FAULT_CHECK_PENDING); // Clear check flag

if (spi_status == HAL_OK && (fault_status & DRV8873_FAULT_BITS_OCP))

{

oc_fault_detected = true;

// Optional: Log specific fault bits 'fault_status'

}

// Optional: Re-enable EXTI line if disabled in ISR

// HAL_NVIC_EnableIRQ(EXTIx_IRQn);

}

// Check Motor 2 if pending

if (READ_BIT(app_flags, MOTOR2_FAULT_CHECK_PENDING))

{

spi_status = Motor2_Read_Register(DRV8873_REG_FAULT, &fault_status);

ATOMIC_CLEAR_BIT(app_flags, MOTOR2_FAULT_CHECK_PENDING); // Clear check flag

if (spi_status == HAL_OK && (fault_status & DRV8873_FAULT_BITS_OCP))

{

oc_fault_detected = true;

// Optional: Log specific fault bits 'fault_status'

}

// Optional: Re-enable EXTI line

// HAL_NVIC_EnableIRQ(EXTIy_IRQn);

}

if (oc_fault_detected)

{

ATOMIC_SET_BIT(app_flags, MOTOR_OVERCURRENT_FAULT_FLAG); // Set specific OC fault flag

CurrentOpState = LIFT_FAULT; // Transition to fault state immediately

// Ensure general fault flag is also set if needed by other logic

ATOMIC_SET_BIT(app_flags, MOTOR_FAULT_FLAG);

return true; // Fault handled

}

// If check flags were set but no OCP detected, it was another fault (UVLO, OTS)

// The general MOTOR_FAULT_FLAG remains set from the ISR.

// Decide how to handle non-OCP faults (e.g., maybe transition to LIFT_FAULT too?)

// For now, we only explicitly transition on OCP.

return false; // No OC fault detected or handled this cycle

}

void AppStart(void)

{

// ... (Initializations: motor_fb_init, gpio_init) ...

MotorInit(); // Calls the function that configures SPI and OCP

// ... (Initial retraction logic) ...

while (1)

{

// --- Check for Faults First ---

// Check if nFAULT triggered and SPI details confirm Overcurrent

if (CheckAndHandleOvercurrentFault()) {

// OC Fault detected and state changed to LIFT_FAULT, skip rest of cycle logic

// The LIFT_FAULT case in the switch statement below will take over.

}

// Handle other general faults (e.g., low speed check from driveActuator)

// This flag might be set by nFAULT ISR (for non-OC faults) or driveActuator

if (READ_BIT(app_flags, MOTOR_FAULT_FLAG) && CurrentOpState != LIFT_FAULT) {

// Decide if *any* fault flag should force LIFT_FAULT state

// CurrentOpState = LIFT_FAULT;

// For now, just signal output low if not already in fault state

HAL_GPIO_WritePin(BS_OUT3_GPIO_Port, BS_OUT3_Pin, GPIO_PIN_RESET);

// Consider clearing the flag here if only signaling, or let LIFT_FAULT handle it.

// ATOMIC_CLEAR_BIT(app_flags, MOTOR_FAULT_FLAG); // Be careful not to clear OC fault

}

// --- Main State Machine ---

switch (CurrentOpState)

{

// ... (Cases: LIFT_INIT to LIFT_RETRACTED) ...

// Ensure these states do not run if CurrentOpState became LIFT_FAULT above

case LIFT_FAULT:

// Actions in Fault State

Motor1_Stop(); // Ensure motors are commanded OFF

Motor2_Stop();

ResetPIDSyncState(); // Reset PID controller state

Ledstrip_Off(); // Turn off status LEDs or set specific fault pattern

HAL_GPIO_WritePin(BS_OUT3_GPIO_Port, BS_OUT3_Pin, GPIO_PIN_RESET); // Activate fault output signal

// Fault Recovery Strategy:

// Option 1: Stay here until manual reset / power cycle.

// Option 2: Timed recovery attempt (Not implemented here)

// Option 3: Wait for specific input to clear fault (Not implemented here)

// Example: Allow clearing fault via Cap Touch press ONLY when in fault state

if (READ_BIT(app_flags, CAP_TOUCH_FLAG)) {

ATOMIC_CLEAR_BIT(app_flags, CAP_TOUCH_FLAG); // Consume touch event

// Attempt to clear DRV fault registers (check datasheet procedure)

Motor_Clear_Faults(MOTOR1_IDX);

Motor_Clear_Faults(MOTOR2_IDX);

// Clear software fault flags

ATOMIC_CLEAR_BIT(app_flags, MOTOR_FAULT_FLAG | MOTOR_OVERCURRENT_FAULT_FLAG | MOTOR1_FAULT_CHECK_PENDING | MOTOR2_FAULT_CHECK_PENDING);

// Deactivate fault signal

HAL_GPIO_WritePin(BS_OUT3_GPIO_Port, BS_OUT3_Pin, GPIO_PIN_SET);

// Go back to a safe state (e.g., RETRACTED or INIT for homing)

actuators[MOTOR1_IDX].pulseCounter = actuators[MOTOR1_IDX].totalPulses; // Force retract

actuators[MOTOR2_IDX].pulseCounter = actuators[MOTOR2_IDX].totalPulses;

CurrentOpState = LIFT_RETRACTING; // Initiate a retract/homing sequence

ATOMIC_SET_BIT(app_flags, INIT_RETRACT_FLAG); // Use init retract logic

// Or maybe just go to LIFT_RETRACTED / LIFT_INIT if retract isn't needed

// CurrentOpState = LIFT_RETRACTED;

}

break;

// ... (Cases: TEST_EXTEND to TEST_STOP) ...

} // End switch(CurrentOpState)

// --- Update Motor Speeds (only if not in Fault) ---

if (CurrentOpState < LIFT_FAULT) // Run PID only in operational states

{

AdjustSpeed();

} else {

// Ensure motors stay stopped if in fault state (redundant but safe)

Motor1_Stop();

Motor2_Stop();

}

// --- Process Inputs (conditionally?) ---

// Decide if inputs should be processed while in fault state

// Probably not motor feedback, maybe GPIOs?

if (CurrentOpState != LIFT_FAULT) {

if (READ_BIT(app_flags, MOTOR_FB_READS_FLAG)) {

ATOMIC_CLEAR_BIT(app_flags, MOTOR_FB_READS_FLAG);

Motor_FB_Reads(); // Process pulses

}

}

// Process GPIOs and Cap Touch regardless of state (needed for fault recovery)

if (READ_BIT(app_flags, GPIO_READS_FLAG)) {

ATOMIC_CLEAR_BIT(app_flags, GPIO_READS_FLAG);

GPIO_Reads(); // Process BS_IN1, IR_INT

}

// Process Cap touch separately if needed by fault state recovery

// (Motor_FB_Reads already handles CAP_TOUCH_FLAG based on its index)

} // End while(1)

}

Explanation and Key Points:

- OCP Configuration: MotorInit now includes example SPI writes to configure the OCP threshold and deglitch time. You MUST replace the example register addresses, bit masks, and values with the correct ones from your specific DRV8873 datasheet.

- nFAULT Handling: The HAL_GPIO_EXTI_Callback detects the nFAULT signal going low (active). It sets flags (MOTOR_FAULT_FLAG and MOTORx_FAULT_CHECK_PENDING) for the main loop. It doesn't immediately assume OCP.

- Main Loop Check (CheckAndHandleOvercurrentFault): When a check flag is set, this function reads the actual fault register via SPI for the corresponding motor.

- OCP Detection: It specifically checks if any OCP bits (DRV8873_FAULT_BITS_OCP) are set in the fault register.

- Fault State (LIFT_FAULT): If an OCP fault is confirmed via SPI, the state machine transitions to LIFT_FAULT. In this state, motors are stopped, fault outputs are activated, and the system waits.

- Fault Recovery: A simple example using the capacitive touch button to clear the fault is included in the LIFT_FAULT state. This involves clearing DRV hardware faults (via SPI Motor_Clear_Faults), clearing software flags, and transitioning back to an operational state (like LIFT_RETRACTING for safety). The exact fault clearing procedure depends on the DRV8873 datasheet.

- SPI Functions: Placeholders like Motor1_Read_Register, Motor1_Write_Register, Motor_Clear_Faults are used. You need to implement these correctly in motor.c using HAL SPI functions (HAL_SPI_TransmitReceive, HAL_SPI_Transmit) and managing Chip Select pins.

- Flag Management: Flags are used to communicate between the ISR and the main loop. Fault check flags are cleared once the SPI check is done. Specific (MOTOR_OVERCURRENT_FAULT_FLAG) and general (MOTOR_FAULT_FLAG) fault flags are used to manage the fault state.

- State Protection: Logic is added to prevent AdjustSpeed and Motor_FB_Reads from running when in the LIFT_FAULT state.

Remember to thoroughly test this, especially the SPI communication, fault register interpretation, and the fault recovery logic.