Beam Break sensor check (currently we only check when lift is retracting we want to check on boot, and each time the lift is triggered to extend from the user input on the cap touch button)

Okay, let's modify the code to reuse the core actions of the existing "IR beam break is obstructed" routine when the check fails on first boot or upon cap touch press in the LIFT_INIT state.

The core actions of the obstruction routine during LIFT_RETRACTING are:

- Stop motor movement.

- Set BS_OUT4 HIGH.

- Set the LED Strip to a specific mode (Ledstrip_IR()).

- (Maintain state until clear).

We can create a helper function to encapsulate the common signaling/feedback actions and call it from both contexts (LIFT_INIT check and LIFT_RETRACTING).

1. Create Helper Function Handle_IR_Obstruction:

Add this static function within app.c:

C

// In app.c

// ... (Includes, Defines, Structs, Global Variables ...)

/**

* @brief Performs actions when an IR obstruction is detected.

* @param during_retract True if obstruction detected during LIFT_RETRACTING state,

* False if detected during check in LIFT_INIT state.

*/

static void Handle_IR_Obstruction(bool during_retract)

{

// Action 1: Signal external device via BS_OUT4

HAL_GPIO_WritePin(BS_OUT4_GPIO_Port, BS_OUT4_Pin, GPIO_PIN_SET);

// Action 2: Visual Feedback via LED Strip

if (during_retract) {

// Use the specific IR LED mode when obstructed during retract

if(TIM1->CCR1 != LEDSTRIP_PWM_IR_DUTY) Ledstrip_IR();

} else {

// Use a general 'blocked'/'error' indication when obstructed at start

// For example, turn the LED strip off or use a flashing pattern (if implemented).

Ledstrip_Off(); // Simple: Turn off LED strip to indicate non-ready state.

}

// Action 3 (Stopping Motors) / Action 4 (Maintaining State)

// are handled by the calling function based on the context.

}

// ... (Rest of existing static functions like AdjustSpeed, driveActuator etc.)

2. Modify LIFT_INIT State in AppStart:

Update the LIFT_INIT case to call the helper function.

C

// In AppStart -> while(1) -> switch(CurrentOpState)

case LIFT_INIT:

// Handle BS_OUT2 pulsing HIGH after delay

if (!Counter_250ms) {

HAL_GPIO_WritePin(BS_OUT2_GPIO_Port, BS_OUT2_Pin, GPIO_PIN_SET);

}

// Check for Capacitive Touch press to start lifting

if (READ_BIT(app_flags, CAP_TOUCH_FLAG))

{

ATOMIC_CLEAR_BIT(app_flags, CAP_TOUCH_FLAG); // Consume the touch event

// *** Check IR Sensor Status BEFORE starting lift ***

// READ_BIT == 0 means flag is CLEAR -> Obstruction detected

if (!READ_BIT(app_flags, IR_INT_FLAG))

{

// --- IR BEAM IS BROKEN ---

// Call the obstruction routine (INIT context)

Handle_IR_Obstruction(false);

// Stay in LIFT_INIT state (do not proceed)

}

else

{

// --- IR BEAM IS CLEAR ---

// Ensure obstruction signal is OFF

HAL_GPIO_WritePin(BS_OUT4_GPIO_Port, BS_OUT4_Pin, GPIO_PIN_LOW);

// Normal LED indication for starting lift

Ledstrip_On();

// Set motor directions for extending

actuators[MOTOR1_IDX].direction = EXTENDING;

actuators[MOTOR2_IDX].direction = EXTENDING;

// Transition state to start extending

CurrentOpState = LIFT_EXTENDING;

}

} // End if Cap Touch Pressed

break; // End LIFT_INIT

3. Modify LIFT_RETRACTING State in AppStart:

Update the LIFT_RETRACTING case to use the helper function and adjust the logic slightly for clarity.

C

// In AppStart -> while(1) -> switch(CurrentOpState)

case LIFT_RETRACTING:

// Check current IR status

// READ_BIT == 0 means flag is CLEAR -> Obstruction detected

bool ir_currently_obstructed = !READ_BIT(app_flags, IR_INT_FLAG);

if (ir_currently_obstructed)

{

// --- IR OBSTRUCTION DETECTED ---

// Call the obstruction routine (RETRACT context)

Handle_IR_Obstruction(true); // Sets BS_OUT4 HIGH, Ledstrip_IR()

IR_Check_Counter = IR_CHECK; // Reset the timer for clearing check

// Stop motors if they were running

if (actuators[MOTOR1_IDX].direction != STOPPED) {

actuators[MOTOR1_IDX].prevPulseCounter = 0.0f; // Reset stall detection

actuators[MOTOR2_IDX].prevPulseCounter = 0.0f;

actuators[MOTOR1_IDX].direction = STOPPED;

actuators[MOTOR2_IDX].direction = STOPPED;

// Ensure motors are commanded off (AdjustSpeed won't run if direction is STOPPED)

Motor1_Stop();

Motor2_Stop();

}

}

else // --- IR SENSOR IS CLEAR ---

{

// Ensure obstruction signal is OFF

HAL_GPIO_WritePin(BS_OUT4_GPIO_Port, BS_OUT4_Pin, GPIO_PIN_LOW);

// Check if the path has been clear long enough (counter expired)

if (!IR_Check_Counter)

{

// --- IR PATH CLEAR - RESUME/CONTINUE RETRACT ---

// Set LED to normal ON state

if(TIM1->CCR1 != LEDSTRIP_PWM_ON_DUTY) Ledstrip_On();

// Set direction to retract (if previously stopped by IR)

if (actuators[MOTOR1_IDX].direction == STOPPED) {

actuators[MOTOR1_IDX].direction = RETRACTING;

actuators[MOTOR2_IDX].direction = RETRACTING;

}

// Stall detection logic (only run if supposed to be moving)

if (!Counter_500ms) {

if (actuators[MOTOR1_IDX].prevPulseCounter == actuators[MOTOR1_IDX].pulseCounter &&

actuators[MOTOR2_IDX].prevPulseCounter == actuators[MOTOR2_IDX].pulseCounter)

{

// Stall detected -> Assume fully retracted

HAL_GPIO_WritePin(BS_OUT4_GPIO_Port, BS_OUT4_Pin, GPIO_PIN_LOW); // Ensure LOW on exit

CurrentOpState = LIFT_RETRACTED;

} else {

// Update prev counters for next stall check

actuators[MOTOR1_IDX].prevPulseCounter = actuators[MOTOR1_IDX].pulseCounter;

actuators[MOTOR2_IDX].prevPulseCounter = actuators[MOTOR2_IDX].pulseCounter;

Counter_500ms = 3;

}

}

} else {

// IR is clear, but timer hasn't expired yet.

// Motors remain stopped (from obstruction handler).

// BS_OUT4 is LOW (set above).

// Ledstrip remains in IR mode (set by obstruction handler).

// Do nothing, wait for IR_Check_Counter to expire.

}

}

// Motor commands are handled outside the switch based on direction,

// or need explicit calls here if AdjustSpeed isn't used in this state.

// Let's ensure motors are driven based on current direction:

driveActuator(MOTOR1_IDX, actuators[MOTOR1_IDX].direction, DESIRED_SPEED);

driveActuator(MOTOR2_IDX, actuators[MOTOR2_IDX].direction, DESIRED_SPEED);

break; // End LIFT_RETRACTING

Summary of Changes:

- A new helper function Handle_IR_Obstruction(bool during_retract) is created to manage setting BS_OUT4 HIGH and controlling the LED strip based on context.

- In LIFT_INIT, when the cap touch is pressed, the IR status is checked. If obstructed, Handle_IR_Obstruction(false) is called, and the lift does not start.

- In LIFT_RETRACTING, when an obstruction is detected, Handle_IR_Obstruction(true) is called, the clear timer is reset, and motors are stopped.

- When the IR sensor is clear (else block in LIFT_RETRACTING), BS_OUT4 is explicitly set LOW. The logic then proceeds to check if the path has been clear long enough (!IR_Check_Counter) before resuming motion and normal LED indication.

- Explicit motor drive commands (driveActuator) are added at the end of the LIFT_RETRACTING case to ensure motors are driven according to the direction set within the state logic (either STOPPED or RETRACTING).

This approach reuses the core signaling/feedback logic for IR obstruction while adapting the specific actions (like LED mode and motor control) to the context where the obstruction is detected.