Display not working correctly reset

- Goal: Implement a user-triggered reset routine that performs two actions sequentially:

  - Signal an external device to reset via a new output BS_OUT5.

  - After a delay, trigger a software reset of the STM32 microcontroller itself.

- Trigger Mechanism: How does the user trigger this?

  - A dedicated physical button connected to a GPIO pin?

  - Holding the existing Capacitive Touch button for an extended period?

  - A specific command sequence via another interface (if available)?

  - Let's assume a dedicated physical reset button connected to a GPIO pin (e.g., RESET_BTN_Pin) for clarity. This pin should be configured as an input, likely with a pull-up or pull-down depending on the button wiring, and debounced.

- New Output Pin (BS_OUT5):

  - Define BS_OUT5_Pin and BS_OUT5_GPIO_Port (e.g., PC1, needs definition in main.h / CubeMX).

  - Configure this pin as a GPIO Output Push-Pull.

  - Determine the signal: Should it be a pulse (HIGH then LOW, or LOW then HIGH) or just set HIGH/LOW? A pulse is common for reset signals. Let's assume setting it HIGH triggers the external reset. It should default to LOW.

- Delay Mechanism:

  - Need a delay between signaling BS_OUT5 and resetting the STM32.

  - Cannot use blocking HAL_Delay() easily if we want to maintain responsiveness or if called from an interrupt context (though a reset button handler in the main loop is better).

  - A simple approach for this specific case might be to use HAL_Delay() right before the reset, as the system is about to reset anyway. However, using a timer is cleaner if other tasks need to run briefly.

  - Let's use HAL_Delay() for simplicity in this terminal action sequence. How long? A few milliseconds should be enough for the external device to register the signal. Let's say 100ms. RESET_DELAY_MS = 100.

- STM32 Software Reset:

  - The standard way to trigger a software reset on ARM Cortex-M devices (including STM32) is using the NVIC_SystemReset() function provided by the CMSIS core headers (usually included via stm32g0xx.h -> core_cm0plus.h).

- Implementation Strategy:

  - Hardware: Define and configure RESET_BTN_Pin (Input, with appropriate pull) and BS_OUT5_Pin (Output Push-Pull, default LOW).

  - Debouncing: Add the RESET_BTN_Pin to the existing GPIO debouncing mechanism (gpio_inst array and GPIO_Reads function).

  - New Flag: Define a flag RESET_BTN_FLAG (e.g., 0x100UL). GPIO_Reads will set/clear this flag based on debounced edges. Let's assume pressing the button makes the input LOW (internal pull-up), so the falling edge sets the flag.

  - Main Loop Check: In AppStart's while(1) loop, check for RESET_BTN_FLAG.

  - Reset Sequence: If RESET_BTN_FLAG is set:

    - Clear the flag (ATOMIC_CLEAR_BIT).

    - Set BS_OUT5 HIGH (HAL_GPIO_WritePin).

    - Delay (HAL_Delay(RESET_DELAY_MS)).

    - Trigger software reset (NVIC_SystemReset()).

- Code Implementation Steps:

  - Defines: Add BS_OUT5_..., RESET_BTN_..., RESET_BTN_FLAG, RESET_DELAY_MS.

  - GPIO Initialization: Ensure pins are configured in CubeMX / main.c. Initialize BS_OUT5 to LOW.

  - Debouncing Setup: Add RESET_BTN to gpio_inst:

C

// In app.c

#define RESET_BTN_IDX 2 // Assuming BS_IN1=0, IR_INT=1

// Increase array size

btn_instance_t gpio_inst[3] = { // Size is now 3

{.state = 0xFF, .port = (uint32_t *)BS_IN1_GPIO_Port, .pin = BS_IN1_Pin,},

{.state = 0x00, .port = (uint32_t *)IR_INT_GPIO_Port, .pin = IR_INT_Pin,},

// Add Reset Button (Assuming Active Low Button -> Initial state FF)

{.state = 0xFF, .port = (uint32_t *)RESET_BTN_GPIO_Port, .pin = RESET_BTN_Pin,}

};

// Update count

btn_init_t gpio_init = {.count = 3, .port_read = (port_read_cb_t)&HAL_GPIO_ReadPin}; // Count is now 3

  - Modify GPIO_Reads: Add handling for RESET_BTN_IDX to set/clear RESET_BTN_FLAG on the appropriate edge (e.g., falling edge if active low button).

  - Modify AppStart Main Loop: Add the check for RESET_BTN_FLAG and the reset sequence. Place this check early in the loop, perhaps before the state machine, so it's highly responsive.

- Write the Code:

  - Add defines.

  - Update gpio_inst and gpio_init.

  - Update GPIO_Reads.

  - Add initialization for BS_OUT5.

  - Add the reset check/sequence in AppStart. Ensure NVIC_SystemReset() is available (it should be via main.h includes).

- Review and Refine:

  - Is the active level/edge detection for the reset button correct? (Assuming active low button, falling edge triggers).

  - Is the delay appropriate?

  - Is the placement of the check in the main loop okay? Putting it early ensures it overrides other operations.

  - Does NVIC_SystemReset() require any specific includes? (Usually handled by default STM32 HAL includes).

  - What happens after reset? The code restarts from the beginning, MotorInit runs, the initial retract sequence runs, etc. This is the expected behavior of a system reset.

C

// ==================================================

// Step 1: Add Defines (in main.h or app.c)

// ==================================================

// --- Reset Button Defines ---

// Choose an unused GPIO pin for the reset button input

#define RESET_BTN_Pin GPIO_PIN_z       // Example: GPIO_PIN_10

#define RESET_BTN_GPIO_Port GPIOy      // Example: GPIOB

// ---

// --- BS_OUT5 Defines ---

// Choose an unused GPIO pin for the external reset output signal

#define BS_OUT5_Pin GPIO_PIN_y         // Example: GPIO_PIN_1

#define BS_OUT5_GPIO_Port GPIOx        // Example: GPIOC

// ---

// --- Flags and Constants ---

// Add a new flag for the reset button press

#define RESET_BTN_FLAG 0x100UL         // Choose an unused flag bit

#define RESET_DELAY_MS 100             // Delay (ms) between signaling external device and resetting MCU

// ---

// ==================================================

// Step 2: Update GPIO Debouncing Setup (in app.c)

// ==================================================

// Define index for the reset button in the gpio_inst array

#define BS_IN1_IDX 0

#define IR_INT_IDX 1

#define RESET_BTN_IDX 2 // New index for the reset button

// Increase size of gpio_inst array to 3

btn_instance_t gpio_inst[3] = {

{.state = 0xFF, .port = (uint32_t *)BS_IN1_GPIO_Port, .pin = BS_IN1_Pin,}, // Assumes Active High input

{.state = 0x00, .port = (uint32_t *)IR_INT_GPIO_Port, .pin = IR_INT_Pin,}, // Assumes Active Low input or interrupt line

// Add Reset Button instance: Assuming Active Low button (connected to GND when pressed)

// Requires internal or external Pull-up resistor. Initial state 0xFF.

{.state = 0xFF, .port = (uint32_t *)RESET_BTN_GPIO_Port, .pin = RESET_BTN_Pin,}

};

// Update the count in gpio_init

btn_init_t gpio_init = {.count = 3, .port_read = (port_read_cb_t)&HAL_GPIO_ReadPin}; // Count is now 3

// ==================================================

// Step 3: Modify GPIO_Reads Function (in app.c)

// ==================================================

/**

* @brief GPIO_Reads acts as shift registers for GPIOs

*/

static void GPIO_Reads(void)

{

uint8_t idx;

for (idx = 0; idx < gpio_init.count; idx++)

{

// Shift in the current pin state

gpio_init.instance[idx].state = (gpio_init.instance[idx].state << 1) | (gpio_init.port_read(gpio_init.instance[idx].port, gpio_init.instance[idx].pin));

// Check for stable low state (Falling Edge detection for Active Low buttons/signals)

if (gpio_init.instance[idx].state == DEBOUNCE_STATE_FALLING_EDGE) // e.g., 0x80 (10000000)

{

if (idx == BS_IN1_IDX) {

// Behavior depends on whether BS_IN1 is active low

ATOMIC_CLEAR_BIT(app_flags, BS_IN1_FLAG); // Assuming Active High, so falling = inactive

} else if (idx == IR_INT_IDX) {

// IR_INT often active low -> Falling edge means beam *broken*

ATOMIC_CLEAR_BIT(app_flags, IR_INT_FLAG);

} else if (idx == RESET_BTN_IDX) {

// Active Low Button: Falling edge means PRESSED

ATOMIC_SET_BIT(app_flags, RESET_BTN_FLAG);

}

}

// Check for stable high state (Rising Edge detection for Active Low buttons/signals)

else if (gpio_init.instance[idx].state == DEBOUNCE_STATE_RISING_EDGE) // e.g., 0x7F (01111111)

{

if (idx == BS_IN1_IDX) {

ATOMIC_SET_BIT(app_flags, BS_IN1_FLAG); // Assuming Active High, so rising = active

} else if (idx == IR_INT_IDX) {

// IR_INT often active low -> Rising edge means beam *clear*

ATOMIC_SET_BIT(app_flags, IR_INT_FLAG);

} else if (idx == RESET_BTN_IDX) {

// Active Low Button: Rising edge means RELEASED

ATOMIC_CLEAR_BIT(app_flags, RESET_BTN_FLAG); // Clear flag on release

}

}

}

}

// ==================================================

// Step 4: Modify AppStart Function (in app.c)

// ==================================================

// Include HAL header for HAL_Delay, NVIC_SystemReset should be implicitly available

#include "stm32g0xx_hal.h" // Or just main.h if it includes HAL

void AppStart(void)

{

// ... (Initializations: motor_fb_init, gpio_init) ...

MotorInit();

// --- Initialize BS_OUT4 and BS_OUT5 state ---

HAL_GPIO_WritePin(BS_OUT4_GPIO_Port, BS_OUT4_Pin, GPIO_PIN_LOW);

HAL_GPIO_WritePin(BS_OUT5_GPIO_Port, BS_OUT5_Pin, GPIO_PIN_LOW); // Initialize BS_OUT5 LOW

// ---

// ... (Initial retraction logic) ...

while (1)

{

// --- Check for User Reset FIRST ---

// Check if the debounced reset button flag is set (button was pressed)

if (READ_BIT(app_flags, RESET_BTN_FLAG))

{

ATOMIC_CLEAR_BIT(app_flags, RESET_BTN_FLAG); // Consume the flag

// --- Start Reset Sequence ---

// 1. Signal external device to reset

HAL_GPIO_WritePin(BS_OUT5_GPIO_Port, BS_OUT5_Pin, GPIO_PIN_SET);

// 2. Delay to allow external device to process the signal

HAL_Delay(RESET_DELAY_MS); // Blocking delay okay here as we are resetting

// 3. Trigger STM32 Software Reset

NVIC_SystemReset(); // This function does not return.

// --- Code below NVIC_SystemReset() will not be executed ---

}

// --- Check for Faults ---

// ... (existing fault check logic using CheckAndHandleOvercurrentFault()) ...

// --- Handle other general faults ---

// ... (existing general fault handling) ...

// --- Main State Machine ---

// Run state machine ONLY if not resetting

switch (CurrentOpState)

{

// ... (All existing states: LIFT_INIT to LIFT_FAULT, TEST states) ...

// Ensure no blocking delays are added inside the state machine itself.

} // End switch(CurrentOpState)

// --- Update Motor Speeds (only if not in Fault) ---

// ... (existing motor speed update logic) ...

// --- Process Inputs ---

// Process flags set by timer interrupts for input reads

if (READ_BIT(app_flags, MOTOR_FB_READS_FLAG)) {

ATOMIC_CLEAR_BIT(app_flags, MOTOR_FB_READS_FLAG);

if (CurrentOpState != LIFT_FAULT) { // Process feedback only if not faulted

Motor_FB_Reads();

}

}

if (READ_BIT(app_flags, GPIO_READS_FLAG)) {

ATOMIC_CLEAR_BIT(app_flags, GPIO_READS_FLAG);

GPIO_Reads(); // Process GPIOs (including Reset button)

}

} // End while(1)

}

Explanation and Important Notes:

- Pin Definitions: You must replace the placeholder pin/port defines (GPIO_PIN_y, GPIOx, GPIO_PIN_z, GPIOy) with actual, available, and correctly configured GPIOs for BS_OUT5 and RESET_BTN.

- GPIO Configuration: Ensure RESET_BTN_Pin is configured as Input (with internal or external Pull-up resistor if using an active-low button) and BS_OUT5_Pin is configured as Output Push-Pull in your CubeMX / main.c MX_GPIO_Init() function.

- Debouncing: The reset button input is added to the existing debouncing structure (gpio_inst, gpio_init, GPIO_Reads).

- Active Level: The code assumes an Active Low reset button (connects to Ground when pressed, pulled HIGH otherwise). If your button is Active High, you'll need to:

  - Initialize its state in gpio_inst to 0x00.

  - Set the RESET_BTN_FLAG on the DEBOUNCE_STATE_RISING_EDGE (0x7F) and clear it on the DEBOUNCE_STATE_FALLING_EDGE (0x80) inside GPIO_Reads.

- GPIO_Reads Logic: Carefully review the logic for all signals within GPIO_Reads to ensure the correct flags (BS_IN1_FLAG, IR_INT_FLAG, RESET_BTN_FLAG) are being set/cleared on the appropriate debounced edges based on whether those signals are Active High or Active Low. The example assumes BS_IN1=ActiveHigh, IR_INT=ActiveLow, RESET_BTN=ActiveLow.

- Main Loop Check: The check for RESET_BTN_FLAG is placed at the beginning of the while(1) loop so it takes priority over other operations.

- Reset Sequence: When the flag is detected, BS_OUT5 is set HIGH, a HAL_Delay() provides a brief pause, and then NVIC_SystemReset() is called.

- NVIC_SystemReset(): This function forces a reset of the microcontroller core and peripherals, causing the code execution to restart from the beginning (similar to a power cycle or pressing the hardware reset pin). No code after this call will be executed in the current run.

- Blocking Delay: HAL_Delay() is used here. While generally discouraged in main loops, it's acceptable immediately before a system reset because normal operation is ceasing anyway.